AI MARITIME INSIGHTS

2026 CARGO SECURING CIC
ONBOARD READINESS CHECKLISTS

General Checklist + Container-Ship Additional Checks | Pre-Release

30 general checks + 12 additional container-ship checks

Purpose: One compact onboard working document comprising a 30-point general cargo-securing checklist and a separate 12-point container-ship addendum. Status: Independent pre-release readiness aid. Cargo Securing is confirmed as the 2026 joint Paris/Tokyo MoU CIC topic. The official questionnaire and final campaign instructions have not yet been published. Review and update this checklist when issued. Basis: Paris/Tokyo MoU confirmation of Cargo Securing as the 2026 joint CIC topic; SOLAS regulations VI/5 and VII/5; CSS Code; MSC.1/Circ.1353/Rev.2; SOLAS VI/2 for container VGM; SOLAS V/31 and V/32 for container-loss reporting; and the official 2016 Tokyo MOU Cargo Securing Arrangements CIC report/questionnaire as a historical benchmark. Important: This is not an official Paris MoU, Tokyo MOU or IMO checklist. The approved ship-specific CSM, applicable Flag/RO/Class requirements and Company SMS remain controlling.

How to use

Tick YES only when the item has been verified and objective evidence is available where applicable.

Tick NO for a gap, defect, missing evidence, uncertainty or uncontrolled temporary condition.

Tick N/A only after confirming the item genuinely does not apply to the ship, cargo or arrangement.

Every NO answer requires closure, escalation or a formally controlled temporary measure.

Do not backdate, recreate or overstate records; physical condition and documentary evidence must agree.

Checklist content

| Checklist | Checks | Application |
| --- | --- | --- |
| Checklist 1 — General onboard readiness | 30 | All applicable ships |
| Checklist 2 — Container-ship additional checks | 12 | Complete in addition |
| CONTAINER-SHIP TOTAL | 42 | Complete both checklists |

CHECKLIST 1 — GENERAL ONBOARD CARGO SECURING READINESS

Complete this checklist for the cargo-securing arrangements applicable to the ship and voyage. Use N/A only where the item genuinely does not apply.

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Approved CSM, cargo plan and assessment |  |  |  |  |  |  |  |
| 1.1 | Approved CSM onboard | A current, Administration-approved and ship-specific Cargo Securing Manual is readily available. | Locate the controlled copy and approval evidence; escalate immediately if it is unavailable. |  |  |  |  |
| 1.2 | Identity, revision and applicability verified | Ship name/IMO number, approval details, revision status and applicability are correct for this vessel and cargo. | Replace obsolete or wrong-vessel copies and resolve any applicability uncertainty. |  |  |  |  |
| 1.3 | Manual complete and current | Required drawings, appendices, cargo-specific instructions and securing-device information reflect actual arrangements. | Obtain missing approved information and formally address any vessel/manual mismatch. |  |  |  |  |
| 1.4 | Cargo information and plan available | Required weights, dimensions, centre of gravity and other securing-relevant information support the actual cargo plan. | Obtain or verify missing information before accepting or securing the cargo. |  |  |  |  |
| 1.5 | Securing arrangement follows the CSM | Stowage position, securing method, securing points, device type, number and capacity comply with the approved CSM. | Correct the arrangement or obtain formal guidance before departure. |  |  |  |  |
| 1.6 | Required assessment completed | Applicable calculations or assessments use the actual cargo, stowage and loading condition, including special-cargo controls. | Recalculate or reassess after any material change; resolve non-compliance before sailing. |  |  |  |  |
| 2. Fixed and portable securing equipment |  |  |  |  |  |  |  |
| 2.1 | Fixed fittings serviceable | Padeyes, D-rings, sockets, foundations and other fixed fittings show no unsafe damage or deterioration. | Mark out, isolate and formally assess or repair defective fittings. |  |  |  |  |
| 2.2 | Portable equipment serviceable | Lashings, chains, wires, rods, turnbuckles, twistlocks and associated gear are fit for intended use. | Remove defective equipment from service and replace it. |  |  |  |  |
| 2.3 | No harmful deterioration | No excessive corrosion, wear, cracking, deformation, seized parts or damaged threads affects safe use. | Clean and inspect suspect equipment; renew it or obtain competent assessment. |  |  |  |  |
| 2.4 | Components compatible and adequate | Connected devices are compatible in type, size, capacity and operating direction for the approved arrangement. | Correct mixed, reversed, forced or otherwise unsuitable connections. |  |  |  |  |
| 2.5 | Identification and evidence traceable | Equipment identification, inventory and certification/inspection evidence are traceable where required by CSM, SMS, Flag or Class. | Reconcile records and markings; do not invent, recreate or backdate evidence. |  |  |  |  |
| 2.6 | Quantity and rejected-equipment control effective | Sufficient serviceable gear is available, while damaged or rejected equipment is identified and segregated. | Replenish shortages and quarantine rejected equipment against unintended reuse. |  |  |  |  |
| 3. Inspection, maintenance, repairs and defects |  |  |  |  |  |  |  |
| 3.1 | Required inspections current | Specified inspections of fixed and portable securing equipment are completed at the required intervals. | Complete overdue inspections and record the actual findings. |  |  |  |  |
| 3.2 | Maintenance effective | Lubrication, freeing-up, corrosion control and other required maintenance keep equipment operational. | Complete necessary maintenance and recheck operation. |  |  |  |  |
| 3.3 | Records match physical condition | CSM/SMS/PMS records, inventories and inspection evidence agree with the condition found onboard. | Correct genuine record errors; never recreate evidence for work not performed. |  |  |  |  |
| 3.4 | Defects properly controlled | Open defects are reported, assessed, risk controlled, assigned and subject to a credible closure plan. | Escalate safety-critical defects and apply controlled restrictions where required. |  |  |  |  |
| 3.5 | Repairs and temporary arrangements authorized | Repairs, welding, replacements, modifications and temporary measures have the required technical basis and approval. | Stop use of unauthorized arrangements and obtain Company/Flag/RO/Class direction. |  |  |  |  |
| 3.6 | Previous findings and overdue actions closed | Relevant PSC, Class, Flag, audit and Company findings have been physically verified without recurrence. | Close remaining actions and retain genuine supporting evidence. |  |  |  |  |

CHECKLIST 1 — GENERAL ONBOARD CARGO SECURING READINESS (CONTINUED)

Continue with crew, operational, voyage and pre-arrival checks. Every NO answer requires closure, escalation or a formally controlled measure.

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4. Crew familiarity, safe access and lashing operations |  |  |  |  |  |  |  |
| 4.1 | Responsible officers familiar | Master and person in charge can locate and explain the applicable CSM arrangements, limitations and checks. | Conduct a focused CSM review and verify understanding. |  |  |  |  |
| 4.2 | Crew can demonstrate practical checks | Assigned crew can select, inspect, fit and verify the securing equipment used onboard. | Provide practical familiarization using the actual equipment and arrangement. |  |  |  |  |
| 4.3 | Duties and supervision clear | Responsibility for planning, supervision, final inspection, voyage monitoring and escalation is understood. | Clarify assignments, communications and the responsible final checker. |  |  |  |  |
| 4.4 | Safe access maintained | Routes, platforms, lighting, guardrails and fall-protection arrangements permit safe securing work and inspection. | Restore access and protective measures before work or inspection. |  |  |  |  |
| 4.5 | PPE, tools and stop-work arrangements effective | Suitable PPE, tools, communications and stop-work authority are available and used. | Correct shortages or unsafe practices before lashing work continues. |  |  |  |  |
| 4.6 | Final departure inspection completed | The installed securing arrangement is physically checked after cargo operations and before departure. | Close missing, loose, incorrectly fitted or unsuitable securing devices. |  |  |  |  |
| 5. Voyage monitoring and pre-arrival close-out |  |  |  |  |  |  |  |
| 5.1 | Voyage checks completed | Cargo securing is checked at suitable intervals and after weather or events likely to affect it. | Complete and record the next required check; correct unsafe conditions promptly. |  |  |  |  |
| 5.2 | Securing adjusted when required | Loose, changed or failed securing arrangements are corrected whenever safe access permits. | Re-tighten or replace equipment and escalate unsafe access or repeated failure. |  |  |  |  |
| 5.3 | Heavy-weather precautions understood | Master and officers understand applicable CSM/CSS precautions for the expected voyage conditions. | Review weather, route, ship-handling and cargo-check actions before exposure. |  |  |  |  |
| 5.4 | Cargo-shift or lashing-failure response ready | Procedures and communications are understood for shifted cargo, failed lashings and unsafe access. | Brief the response and confirm necessary equipment and escalation routes. |  |  |  |  |
| 5.5 | Pre-arrival deck walk completed | A focused inspection confirms physical condition, housekeeping, safe access and consistency with current records. | Correct findings and retain honest, current evidence where required. |  |  |  |  |
| 5.6 | Every identified gap closed or controlled | Every NO answer is closed or formally controlled and escalated before arrival, with Master verification. | Do not sign off an uncontrolled safety-critical item. |  |  |  |  |

CHECKLIST 2 — ADDITIONAL CHECKS FOR CONTAINER SHIPS

Container ships complete these 12 additional checks after Checklist 1. The approved ship-specific CSM and CSAP remain controlling.

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C1. Container planning, stowage and lashing assessment |  |  |  |  |  |  |  |
| C1 | Cargo Safe Access Plan verified | The approved CSAP is available where required and agrees with the vessel's actual access arrangements. | Correct unsafe access and resolve any plan/arrangement mismatch through the proper approval route. |  |  |  |  |
| C2 | VGM available and used | Verified Gross Mass is available before loading and is used in preparation of the ship's stowage plan. | Resolve missing or inconsistent VGM data under the applicable procedure. |  |  |  |  |
| C3 | Final bay plan matches the vessel | Final container positions, weights and special units agree with the actual stow onboard. | Correct discrepancies and reissue the controlled final plan. |  |  |  |  |
| C4 | Stack, tier and loading limits respected | Stack weights, tier limits and other permissible loading limits are not exceeded. | Re-stow or recalculate any non-compliant stack before departure. |  |  |  |  |
| C5 | Ship-specific lashing assessment completed | The CSM-required method uses the correct ship configuration, final container weights, positions and loading condition. | Correct inputs and repeat the assessment after material changes. |  |  |  |  |
| C6 | Outputs and special arrangements controlled | No warning or overload is bypassed; results are available and special or non-standard units follow applicable instructions. | Restow, revise the arrangement or obtain formal technical guidance before sailing. |  |  |  |  |
| C2. Container equipment, final arrangement and loss reporting |  |  |  |  |  |  |  |
| C7 | Cell guides and foundations serviceable | Cell guides, container foundations, hatch-area interfaces and surrounding structure show no unsafe condition. | Report, assess, repair or restrict affected positions. |  |  |  |  |
| C8 | Lashing bridges and deck fittings serviceable | Lashing bridges, platforms, sockets, D-rings and associated supports remain structurally and operationally sound. | Restrict access or use and obtain technical assessment for defects. |  |  |  |  |
| C9 | Twistlocks serviceable and suitable | Twistlocks are correct for the intended position, operate freely and lock as designed. | Remove seized, damaged, mixed or unsuitable units from service. |  |  |  |  |
| C10 | Rods and turnbuckles serviceable | Lashing rods, turnbuckles and securing bars are of the correct type and length with sound threads, pins and end fittings. | Clean, lubricate, repair or replace defective or incompatible equipment. |  |  |  |  |
| C11 | Final lashing arrangement verified | Installed lashings and twistlocks match the final bay plan, approved CSM pattern and completed assessment. | Correct missing, loose, reversed or wrongly positioned devices. |  |  |  |  |
| C12 | Container-loss reporting readiness confirmed | Bridge and Master know the SOLAS V/31 and V/32 reporting procedure for containers lost at sea. | Verify reporting information, templates, contacts and responsible persons. |  |  |  |  |