AI MARITIME INSIGHTS

PSC MACHINERY & MAINTENANCE
READINESS CHECKLIST

Enhanced Machinery Inspection / MDI Trigger Version

151 YES / NO / N/A check items for vessel and shore-side pre-arrival preparation

Purpose: A practical readiness checklist to help vessels and shore teams verify machinery maintenance evidence before PSC exposure. Basis: Developed from IMO III 12/INF.21, Guidance for port State control officers on inspection for ship's machinery and maintenance. The source guidance addresses document/log review, initial machinery inspection, maintenance-system verification, clear grounds for more detailed inspection, main engine alarms, main engine maintenance, starting systems, power generation, electrical systems, steering gear, boilers, UMS and reporting/corrective-action expectations. Important: This is not an official IMO checklist and does not replace Class, Flag, Company SMS, statutory certification or PSC requirements.

How to use

- Tick YES only when the item has been verified and evidence is available where applicable.

- Tick NO where there is a gap, defect, missing evidence, uncertainty or uncontrolled temporary condition.

- Tick N/A only where the item genuinely does not apply to the ship or equipment arrangement.

- Every NO answer requires closure, evidence, escalation or a controlled temporary measure before arrival.

- Do not hide defects through unjustified alarm suppression, breaker isolation, closed valves, jumper cables or undocumented temporary repairs.

Checklist content

| Section | Items | Completion status |
| --- | --- | --- |
| 1. Documents, certificates, logs and records | 10 | Complete / Pending |
| 2. PMS and maintenance-system evidence | 8 | Complete / Pending |
| 3. Engine-room general condition | 12 | Complete / Pending |
| 4. Clear grounds / MDI trigger screen | 10 | Complete / Pending |
| 5. Main engine alarm systems | 9 | Complete / Pending |
| 6. Main engine component maintenance | 11 | Complete / Pending |
| 7. Main engine starting systems | 12 | Complete / Pending |
| 8. Power generation | 14 | Complete / Pending |
| 9. Electrical systems and emergency power | 13 | Complete / Pending |
| 10. Steering gear | 14 | Complete / Pending |
| 11. Boilers and steam systems | 13 | Complete / Pending |
| 12. UMS / periodically unattended machinery spaces | 15 | Complete / Pending |
| 13. Defect reporting and pre-arrival corrective action | 10 | Complete / Pending |
| TOTAL | 151 | 151-point checklist |

1. Documents, certificates, logs and records

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | SAFCON valid and endorsed | Confirm SOLAS Safety Construction Certificate is valid, endorsed and available. | Keep certificate ready for PSC review. |  |  |  |  |
| 1.2 | Class Status Report reviewed | Check outstanding machinery-related recommendations, conditions, memoranda or overdue items. | Close, update or obtain Class clarification before arrival. |  |  |  |  |
| 1.3 | Flag / RO conditions declared | Check exemptions, Conditions of Authority, Conditions of Class, equivalence letters or temporary permissions affecting propulsion or auxiliary machinery. | Prepare copies and ensure Master / Chief Engineer can explain status. |  |  |  |  |
| 1.4 | Current machinery defects declared internally | Confirm whether any machinery is inoperative, under maintenance, damaged or temporarily repaired. | Prepare defect report, risk assessment, temporary measures and repair plan. |  |  |  |  |
| 1.5 | Oil Record Book checked | Verify OWS use, sludge/bilge handling entries and sludge capacity records are consistent. | Correct factual clerical errors only as per procedure; do not recreate records. |  |  |  |  |
| 1.6 | NOx Technical Files checked | Confirm availability and consistency with engine components, settings and spare parts where applicable. | Resolve mismatch with Superintendent / Class before inspection exposure. |  |  |  |  |
| 1.7 | Fuel change-over records complete | Check ECA transition entries, times, positions, fuel grades and temperatures. | Complete missing supporting evidence before arrival. |  |  |  |  |
| 1.8 | BDNs available | Bunker Delivery Notes available and consistent with fuel records. | Keep latest BDNs ready. |  |  |  |  |
| 1.9 | SEEMP II / III documentation available | Statement of Compliance and relevant reporting year/data available. | Keep documents ready for review. |  |  |  |  |
| 1.10 | Engine logbook reviewed | Machinery operation, alarms, failures, repairs and tests are properly recorded. | Resolve gaps and prepare explanation for any abnormal event. |  |  |  |  |

2. PMS and maintenance-system evidence

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1 | PMS is up to date | Planned Maintenance System reflects current machinery and active equipment onboard. | Update overdue or incorrect jobs only with factual evidence. |  |  |  |  |
| 2.2 | PMS records match physical condition | PMS jobs, engine logbook and actual machinery condition are consistent. | Investigate any completed PMS job where condition suggests otherwise. |  |  |  |  |
| 2.3 | Manuals and technical documents available | Equipment operation and maintenance manuals onboard; documents match actual model/type. | Replace wrong manuals or obtain correct maker documentation. |  |  |  |  |
| 2.4 | Crew understand PMS use | Engineers can explain maintenance recording, reporting and defect escalation. | Brief engineers; verify they can demonstrate the process. |  |  |  |  |
| 2.5 | Work permits issued where required | Ongoing maintenance covered by permit, isolation and lock-out/tag-out where applicable. | Close unsafe maintenance arrangements before arrival. |  |  |  |  |
| 2.6 | LOTO equipment available and used | Lock-out equipment available and applied during relevant maintenance. | Correct immediately if missing or not used. |  |  |  |  |
| 2.7 | Maintenance failures reviewed | Recent breakdowns assessed to confirm whether caused by missed or ineffective maintenance. | Prepare RCA / corrective action if maintenance failure is suspected. |  |  |  |  |
| 2.8 | Critical spares assessed | Company/vessel has assessed spare parts required for machinery whose failure may create hazardous situations. | Confirm critical spare status and shortages before arrival. |  |  |  |  |

3. Engine-room general condition

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.1 | Cleanliness acceptable | No excessive oil stains, oily bilges, oily drip trays, oil-soaked lagging or oily rags. | Clean, rectify leak source and document action taken. |  |  |  |  |
| 3.2 | No temporary save-alls hiding leakage | Temporary trays/save-alls are not used as a substitute for repair. | Repair leakage or prepare controlled temporary measure with shore approval. |  |  |  |  |
| 3.3 | Ventilation effective | Engine-room ventilation operational and atmosphere safe for work. | Rectify defective fans/dampers before arrival. |  |  |  |  |
| 3.4 | Lighting adequate | Lighting operational in work areas, access areas and machinery spaces. | Replace failed lights and secure fittings. |  |  |  |  |
| 3.5 | Electrical condition acceptable | No visible unsafe electrical condition; insulation records normal; no serious earth fault. | Investigate low insulation / earth faults before arrival. |  |  |  |  |
| 3.6 | Piping systems in good order | Control valves, quick-closing valves, pipe joints, relief valves and associated equipment in good condition. | Rectify leaks, seized valves or damaged fittings. |  |  |  |  |
| 3.7 | ECR panel / monitoring normal | No unexplained abnormal alarms, suppressed alarms or serious monitoring issues. | Print/review alarm history and close unexplained items. |  |  |  |  |
| 3.8 | Machinery condition acceptable | Main engine, auxiliaries, pumps and systems free of cracks, corrosion, leakage, abnormal wear or hidden isolations. | Rectify; do not hide leakage by closing isolation valves. |  |  |  |  |
| 3.9 | Escape routes clear | No stores, spares, waste or equipment blocking escape routes. | Clear routes immediately. |  |  |  |  |
| 3.10 | Gauges readable and normal | Gauges undamaged; readings normal, including starting-air pressure. | Replace defective gauges and investigate abnormal readings. |  |  |  |  |
| 3.11 | Rubber expansion joints acceptable | REJs/non-metallic expansion joints visually satisfactory and within replacement/survey requirements. | Replace or obtain Class advice where condition/date is doubtful. |  |  |  |  |
| 3.12 | Boiler basic condition acceptable | Two water-level indicators where required; safety systems not obstructed. | Rectify before arrival if defective. |  |  |  |  |

4. Clear grounds / MDI trigger screen

Note: YES means the potential MDI trigger is absent, controlled or satisfactorily closed. Any NO answer in this section should be escalated before arrival.

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.1 | No recent propulsion failure / lack of main engine power | No recent propulsion loss, inability to start, reverse, stop or regulate speed, unless fully repaired/controlled. | If not satisfactory, notify office; assess Class/Flag involvement; prepare repair evidence. |  |  |  |  |
| 4.2 | Required generators available | Required auxiliary generators available; required power and load sharing are adequate. | Test generators; verify load capacity and corrective action. |  |  |  |  |
| 4.3 | No excessive oil leakage | No oil leakage, oil-soaked lagging, oil in bilges or save-alls indicating poor maintenance. | Stop source, clean area, record repair. |  |  |  |  |
| 4.4 | Crew can demonstrate required tests | Crew can demonstrate equipment tests as per maker/SMS instructions. | Conduct immediate familiarization and record toolbox/training. |  |  |  |  |
| 4.5 | No unjustified suppressed/inhibited alarms | No alarms inhibited, off-scan, reposed, suppressed or unexplained without valid control. | Restore, justify and document with senior engineer and office review. |  |  |  |  |
| 4.6 | Starting air adequate | Receiver pressure adequate and required compressors available/in service. | Restore pressure, test compressors, drain receivers, check leaks. |  |  |  |  |
| 4.7 | No serious earth fault | No large earth fault on 220V and no earth fault on 440V systems. | Locate and rectify; do not hide by simply switching off a faulty circuit. |  |  |  |  |
| 4.8 | Engine room not cluttered | No rubbish, oily rags, unsecured spares or loose parts creating PSC concern. | Clean and secure immediately. |  |  |  |  |
| 4.9 | Automation/alarm/safety systems operational | Main engine, auxiliary engine and boiler automation, alarm and safety systems operational. | Escalate; repair or obtain Class/Flag guidance. |  |  |  |  |
| 4.10 | No uncontrolled emergency repair evidence | No loose tools/parts around critical equipment; recent repairs properly completed and recorded. | Close job properly; prepare repair record and risk control. |  |  |  |  |

5. Main engine alarm systems

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1 | Alarm panels/displays satisfactory | No damage to alarm displays, alarms, UPS or associated components. | Repair defective components. |  |  |  |  |
| 5.2 | Alarm display clear and accurate | Alarm display shows correct alarm description and status. | Rectify unclear or faulty display. |  |  |  |  |
| 5.3 | Alarm history reviewed | Historical alarms reviewed; false alarms and repeated alarms investigated. | Prepare explanation and corrective action. |  |  |  |  |
| 5.4 | Inhibited alarms justified | Manually inhibited alarms have valid reason and authorization. | Restore unjustified inhibited alarms. |  |  |  |  |
| 5.5 | Sensors and connections secure | Sensor appearance and connections satisfactory. | Secure/replace defective sensors. |  |  |  |  |
| 5.6 | Lamp and sound test satisfactory | Centralized alarm panel visual and audible tests completed. | Rectify failed alarm indications. |  |  |  |  |
| 5.7 | Critical alarms tested where safe | Cooling water high temperature, exhaust gas high temperature, power failure, low starting air, HP fuel leak, crankcase oil mist/bearing temperature and auxiliary pump changeover verified where applicable. | Test only under safe conditions and SMS/maker procedure; involve Class if required. |  |  |  |  |
| 5.8 | Alarms audible/visible at required locations | Bridge, machinery space, ECR and required panels receive correct alarm. | Rectify alarm extension or sounder faults. |  |  |  |  |
| 5.9 | ECR alarm records retained | Tested alarms appear in ECR records/history. | Save/print test evidence if useful. |  |  |  |  |

6. Main engine component maintenance

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1 | Drawings/specifications available | Engineering drawings, specifications and technical data available for main engine/components. | Obtain missing documents. |  |  |  |  |
| 6.2 | Main engine jobs included in PMS | Fuel pumps, injectors, exhaust valves, liners, pistons, bearings, remote control, starting, fuel, LO and cooling systems covered. | Update PMS coverage if missing. |  |  |  |  |
| 6.3 | Maintenance records retained | Repair/maintenance records match PMS and logbook. | Correct traceability gaps. |  |  |  |  |
| 6.4 | Main engine external condition acceptable | Clean, no abnormal corrosion, deformation, cracks, leaks, external wiring or unauthorized piping. | Rectify before arrival. |  |  |  |  |
| 6.5 | Oil and water piping secured | Piping, supports and connections in good condition. | Repair loose supports/leaks. |  |  |  |  |
| 6.6 | Instruments/gauges normal | Readings within expected operating range. | Investigate abnormal values. |  |  |  |  |
| 6.7 | Protective functions tested where applicable | Low LO pressure slowdown/shutdown and bearing temperature monitoring verified where applicable. | Test safely under procedure; document result. |  |  |  |  |
| 6.8 | NOx-related spare parts controlled | For applicable engines, spare parts match NOx Technical File requirements. | Check before fitting non-approved components. |  |  |  |  |
| 6.9 | Crew familiar with maintenance | Responsible engineers can explain maintenance procedures for main engine components. | Conduct focused briefing. |  |  |  |  |
| 6.10 | Cleaning/inspection arrangements available | Special tools/access/provisions for cleaning, inspection and maintenance available. | Prepare tools and access arrangements. |  |  |  |  |
| 6.11 | Non-metallic expansion joints controlled | Condition, survey and replacement status verified, especially below deepest load waterline. | Replace or obtain Class advice if doubtful. |  |  |  |  |

7. Main engine starting systems

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1 | Starting-system equipment condition acceptable | Air/electric/hydraulic starting components free of visible damage, corrosion, leakage or ageing. | Repair before arrival. |  |  |  |  |
| 7.2 | Air compressors and receivers clean/serviceable | Protective covers intact; no air leaks; compressors available. | Test compressors and rectify leaks. |  |  |  |  |
| 7.3 | Starting-air valves/components functional | Main/aux air receivers, starting valves, distributors, cylinder valves, flame arresters, safety valves, drains, dryers and reducers checked. | Repair leaking/defective components. |  |  |  |  |
| 7.4 | Bypass and back-up systems controlled | Bypass valves not improperly used; back-up reducers/systems functional. | Remove unsafe bypass condition or document approved temporary measure. |  |  |  |  |
| 7.5 | Turning gear interlock tested | Starting air/electrical start circuit disconnected when turning gear engaged. | Test and record result. |  |  |  |  |
| 7.6 | Crew familiar with starting procedures | Engineers can explain operation and maintenance of starting system. | Brief and verify demonstration. |  |  |  |  |
| 7.7 | Emergency starting system effective | Emergency air compressor/receiver or equivalent arrangement can be activated. | Test before arrival where safe. |  |  |  |  |
| 7.8 | Starting-air gauges and safety devices satisfactory | Gauges correct; safety valves/fusible plugs not leaking/failed. | Rectify defective items. |  |  |  |  |
| 7.9 | Non-return / blocking devices satisfactory | Backfire prevention devices not damaged or failed. | Repair before arrival. |  |  |  |  |
| 7.10 | Overpressure safety valves controlled | 5-year test tags visible; no paint covering; no corrosion. | Clean/verify certification/renew if overdue. |  |  |  |  |
| 7.11 | Starting-air pipeline separation verified | Receiver-to-engine lines separated from compressor discharge-to-receiver lines. | Investigate any non-standard arrangement. |  |  |  |  |
| 7.12 | Air receiver drains operate correctly | Drains open/close freely; oil-water mixture drains without blockage/residue. | Drain, clean and rectify stuck valves. |  |  |  |  |

8. Power generation

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.1 | Generator log history reviewed | Malfunctions and corrective actions recorded and closed. | Prepare explanation for repeated failures. |  |  |  |  |
| 8.2 | Generator PMS implemented | Generator maintenance plan active and followed. | Close overdue critical jobs. |  |  |  |  |
| 8.3 | Generators cycled and load sharing normal | Auxiliary generators rotated properly; load sharing adequate; exhaust temperatures consistent. | Test and adjust before arrival. |  |  |  |  |
| 8.4 | Survey reports/certificates available | Periodic survey reports and certificates available. | Keep ready for inspection. |  |  |  |  |
| 8.5 | Generator visual condition acceptable | Generator, auxiliary systems, pipelines, valves, instruments and equipment free of visible defects. | Rectify leaks/damage. |  |  |  |  |
| 8.6 | Exhaust and lubrication condition satisfactory | No exhaust smoke leaks; turbocharger/governor oil level and condition satisfactory. | Rectify before arrival. |  |  |  |  |
| 8.7 | Fuel oil system safe | Hot surface insulation intact; fuel oil isolation valves functional where applicable. | Repair insulation/isolation defects. |  |  |  |  |
| 8.8 | Lubricating oil system satisfactory | Filter pressures normal; no LO leaks; oil analysis records reviewed. | Investigate abnormal pressure/oil condition. |  |  |  |  |
| 8.9 | Cooling water system satisfactory | No leakage/corrosion; outlet temperatures normal. | Rectify leaks and abnormal temperature. |  |  |  |  |
| 8.10 | Generator starting air system satisfactory | Compressors, receivers, pipelines, safety valves and drains checked. | Drain/repair/test before arrival. |  |  |  |  |
| 8.11 | Safety protection devices functional | Low LO pressure protection and other safety devices verified. | Test safely and record. |  |  |  |  |
| 8.12 | Paralleling and load shedding tested | Synchronization, paralleling and load shedding function correctly. | Test without disrupting cargo/critical operations. |  |  |  |  |
| 8.13 | Capacity with one generator unavailable confirmed | Remaining generators can supply normal propulsion, safety and habitability loads. | Verify against load calculation table. |  |  |  |  |
| 8.14 | Engineer familiar with synchronization/load shedding | Responsible engineer can explain and demonstrate if required. | Conduct briefing/demo. |  |  |  |  |

9. Electrical systems and emergency power

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9.1 | Switchboard area clear | No obstruction around switchboard; safe access maintained. | Clear area immediately. |  |  |  |  |
| 9.2 | Indicator lights and insulation alarms function | Power/lighting insulation alarms operational; low insulation not hidden by switching off breakers without control. | Investigate and rectify low insulation. |  |  |  |  |
| 9.3 | Live-part protection satisfactory | Insulated mats, back panels, handrails and covers in place. | Restore missing protection. |  |  |  |  |
| 9.4 | Electrical grounding satisfactory | Metal casings/frames reliably grounded. | Rectify grounding defects. |  |  |  |  |
| 9.5 | Hazard-area electrical protection satisfactory | Explosion-proof, moisture-proof and anti-corrosion features in place where required. | Repair/replace defective fittings. |  |  |  |  |
| 9.6 | Main power supply stable | Main generators supply sufficient power across all three phases. | Investigate instability/imbalance. |  |  |  |  |
| 9.7 | Emergency source of power tested | Emergency generator starts automatically and supplies required load within required time; batteries provide immediate power where applicable. | Test and record before arrival. |  |  |  |  |
| 9.8 | Emergency generator fuel sufficient | Fuel sufficient for required operating duration; tank gauge and replenishment records checked. | Replenish and record. |  |  |  |  |
| 9.9 | Emergency generator starting capacity adequate | At least three consecutive starts, plus secondary energy source for further starts where required. | Charge/repair batteries/air/hydraulic starting system. |  |  |  |  |
| 9.10 | Overload protection settings indicated | Circuit protective-device rating/settings permanently indicated. | Replace missing labels/settings indication. |  |  |  |  |
| 9.11 | Lighting fixtures secure | Anti-vibration and waterproof protection suitable in vibrating/damp areas. | Secure/replace defective fixtures. |  |  |  |  |
| 9.12 | Enclosure protection satisfactory | Electrical enclosures prevent injury during normal handling/touching. | Restore covers/enclosures. |  |  |  |  |
| 9.13 | Emergency generator load test completed | Emergency fire pump / steering pump or suitable emergency load tested where safe. | Test and document result. |  |  |  |  |

10. Steering gear

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.1 | Steering gear records complete | Calibration/testing records and emergency steering drills at least every three months. | Update evidence and conduct drill if overdue. |  |  |  |  |
| 10.2 | No hydraulic leaks/seepage | Pipe joints, valves, cylinders and containment areas checked. | Repair leaks; clean oil accumulation. |  |  |  |  |
| 10.3 | Hydraulic oil reserve satisfactory | Recharging tank/fixed filling line fitted where required; reserve oil sufficient; level gauge fitted. | Replenish/repair level indication. |  |  |  |  |
| 10.4 | Gauges and vents satisfactory | Air vents and pressure gauges at both cylinder ends installed and working. | Repair defective gauges/vents. |  |  |  |  |
| 10.5 | Tank gauges/self-closing devices correct | Temperature/level gauges working; self-closing devices not held open. | Correct unsafe arrangements. |  |  |  |  |
| 10.6 | Rudder stock sleeve condition checked | No water ingress due to seal failure. | Investigate leakage immediately. |  |  |  |  |
| 10.7 | Linkages and mechanical play acceptable | No loose linkages or excess play affecting course keeping. | Repair/adjust before arrival. |  |  |  |  |
| 10.8 | Steering response satisfactory | Main/auxiliary steering achieve required angle and response time. | Test locally and from bridge. |  |  |  |  |
| 10.9 | Steering power failure alarm tested | Audible/visual alarm activates on power interruption; restart after power restoration verified. | Test and record. |  |  |  |  |
| 10.10 | Limit switches function | Steering gear limit switches tested. | Repair if defective. |  |  |  |  |
| 10.11 | Crew familiar with isolation valves | Cargo ships: crew can explain valve-closing procedures in isolation system. | Brief and demonstrate. |  |  |  |  |
| 10.12 | Bridge-steering communication tested | Communication works; local/bridge rudder angle indicators match. | Repair communication/indicator mismatch. |  |  |  |  |
| 10.13 | Steering alarms functional | Power failure, short circuit, overload and phase failure alarms tested at required locations. | Rectify alarm failures. |  |  |  |  |
| 10.14 | Emergency steering drill completed | Drill demonstrates operational readiness. | Conduct and record before arrival. |  |  |  |  |

11. Boilers and steam systems

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11.1 | Boiler certificates/reports available | Survey certificate, calibration report and boiler logbook available. | Keep ready for PSC. |  |  |  |  |
| 11.2 | Safety valve calibration verified | Calibration of boiler safety valves recorded. | Verify dates / arrange calibration if overdue. |  |  |  |  |
| 11.3 | Boiler water treatment controlled | Boiler water dosing, water-quality tests, shore monitoring and chemicals available where applicable. | Complete missing tests / replenish chemicals. |  |  |  |  |
| 11.4 | Boiler maintenance records complete | Routine maintenance records available and consistent. | Close record gaps. |  |  |  |  |
| 11.5 | Boiler external condition acceptable | No corrosion, deformation, cracks, leaks, smoke/gas leaks on boiler body, burner, exhaust, steam/feedwater lines, valves or manholes. | Repair before arrival. |  |  |  |  |
| 11.6 | Burner and sight glass satisfactory | Burner operable; sight glass clean; no black smoke. | Clean/adjust burner; rectify combustion issue. |  |  |  |  |
| 11.7 | Boiler operates safely under load | Reliable under normal/varying load; alarms, interlocks and automatic controls function. | Test and record. |  |  |  |  |
| 11.8 | Insulation and pipelines satisfactory | Boiler body, fuel oil, steam and feedwater insulation intact; no pipeline leakage. | Repair insulation/leaks. |  |  |  |  |
| 11.9 | Boiler valves operable | Main steam, water sampling, upper/lower drain and steam stop valves in good condition and free to operate. | Free up/repair seized valves. |  |  |  |  |
| 11.10 | Safety valve easing gear satisfactory | Not improperly set, cracked or corroded; outlet leads to safe area; no leakage. | Rectify/seek Class advice if doubtful. |  |  |  |  |
| 11.11 | Unattended boiler safety functions work | Low water level, air supply failure and flame failure trigger alarm and fuel shut-off where applicable. | Test; brief responsible crew. |  |  |  |  |
| 11.12 | Water-level indication compliant | At least two means of indication, including one direct-reading gauge glass where required. | Restore defective gauge. |  |  |  |  |
| 11.13 | Remote safety valve operation verified | Remote operation of safety valves possible where fitted/required. | Test and record. |  |  |  |  |

12. UMS / periodically unattended machinery spaces

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12.1 | UMS notation/procedure verified | Certificate/Class notation and company UMS procedure available and followed. | Keep evidence ready. |  |  |  |  |
| 12.2 | Manning/watch arrangement consistent | Minimum Safe Manning Certificate and engine-room watch arrangement match UMS operation. | Clarify if operating manned instead of UMS. |  |  |  |  |
| 12.3 | Bridge machinery indicators satisfactory | Ahead/astern/stop, RPM and starting-air pressure indications available/readable. | Repair defective indications. |  |  |  |  |
| 12.4 | Sensors/probes condition acceptable | Temperature, smoke/light sensing probes not aged, damaged or poorly connected. | Repair/replace defective probes. |  |  |  |  |
| 12.5 | Visual displays readable | LCD/screens readable; no abnormal symbols/internal communication faults. | Rectify before arrival. |  |  |  |  |
| 12.6 | UMS alarms/monitoring functional | Alarm, monitoring, automatic control, safety interlocks and power supply reliable. | Conduct UMS function test. |  |  |  |  |
| 12.7 | Alarm extensions functional | Visual/audible alarms indicated in ECR, wheelhouse and accommodation panels. | Test and repair extensions. |  |  |  |  |
| 12.8 | Automatic operations functional | Auto start/stop, auto changeover, generator load sharing/load shedding and essential auxiliary functions work. | Test and record. |  |  |  |  |
| 12.9 | Control/safety power supply reliable | Alarm system and essential controls powered by main and emergency sources with reliable changeover. | Test power failure/changeover. |  |  |  |  |
| 12.10 | Personnel/deadman alarm tested where fitted | Reset, pre-warning and personnel alarm timing/function verified. | Test and record. |  |  |  |  |
| 12.11 | Bilge high-level alarm tested | Bilge high-level alarm works; prolonged/frequent bilge pump operation alarm works where fitted. | Test before arrival. |  |  |  |  |
| 12.12 | Flooding valve controls accessible | Sea inlet/discharge/bilge injection controls reachable in time or operable from suitable level. | Verify remote/local operation. |  |  |  |  |
| 12.13 | No locked-out UMS alarms | Central/extended alarms and power-failure alarms operable; no unjustified isolated alarms. | Restore or formally justify. |  |  |  |  |
| 12.14 | Dual pump auto-switching tested | Standby pump starts automatically and alarm triggers when duty pump stops. | Test and record. |  |  |  |  |
| 12.15 | Standby generator automation tested | Automatic start, synchronization, load sharing, de-loading, load distribution and step load shedding operate. | Test under safe conditions. |  |  |  |  |

13. Defect reporting and pre-arrival corrective action

| No. | Check item | Details to verify | Action before arrival | Y | N | N/A | Remarks / evidence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13.1 | Defects affecting safety reported | Defects affecting ship safety or equipment completeness reported to Flag/RO/Class/Port State where required. | Do not wait for PSC to discover it. Report through correct channels. |  |  |  |  |
| 13.2 | Temporary corrective measures controlled | Temporary measures technically assessed, recorded and approved where required. | Prepare risk assessment, repair plan and spare/technician ETA. |  |  |  |  |
| 13.3 | Main engine serious defects escalated | Inability to start/reverse/stop/regulate speed, serious cracks, abnormal parameters, vibration or minimum-speed issue checked. | Treat as critical; involve office/Class before arrival. |  |  |  |  |
| 13.4 | Automation failures controlled | Failed automation affecting UMS or critical operation assessed. | Repair before departure/arrival or switch to controlled manned mode if approved. |  |  |  |  |
| 13.5 | Bridge-engine communication working | Telegraph/voice-powered telephone normal. | Repair before arrival; treat failure as serious. |  |  |  |  |
| 13.6 | Compressed-air defects controlled | Any compressed-air defect affecting other power systems identified. | Rectify before arrival; escalate if affecting propulsion/power. |  |  |  |  |
| 13.7 | Boiler/steam defects controlled | Structural defects, tube cracks, water-pipe leaks, auto water/ignition failures assessed. | Repair; involve RO/Class where structural or operationally serious. |  |  |  |  |
| 13.8 | Steering gear defects controlled | Response time/angle/alarm/leakage issues assessed. | Repair before arrival; severe leakage or failed response may require Class/RO. |  |  |  |  |
| 13.9 | Critical piping defects controlled | Fuel, cooling and hydraulic systems for main engine/steering gear checked. | Rectify to Class/RO satisfaction before departure where required. |  |  |  |  |
| 13.10 | Electrical serious defects controlled | Generator failure, emergency power failure, critical motor failure or jumper cables isolating alarm sensors/indicators checked. | Remove unsafe jumpers; rectify immediately; escalate to office/Class. |  |  |  |  |