AI MARITIME INSIGHTS	● FREE FIELD GUIDE

PRE-ARRIVAL READINESS  ·  IMO III 12/INF.21

Enhanced Machinery &

Maintenance PSC Checklist

A self-verification tool built from the harmonized guidance for Port State Control officers on enhanced machinery & maintenance inspections.

| Initial inspection general condition | Clear grounds §4 triggers | MDI detailed inspection |
| --- | --- | --- |

D  Detainable      R  Rectify before departure      MDI  Clear-grounds trigger

WHAT'S INSIDE    documents & records review, initial-inspection items, clear-grounds triggers, plus:

| 5.4 Main engine alarm systems | 5.5 Main engine components |
| --- | --- |
| 5.6 Starting systems (air / electric) | 5.7 Power generation |
| 5.8 Electrical systems | 5.9 Steering gear |
| 5.10 Boilers | 5.11 Unmanned machinery (UMS) |

Source: IMO Sub-Committee III 12/INF.21 (15 May 2026) — China, Tokyo MoU, Indian Ocean MoU, Caribbean MoU.

Reference guidance only — apply the current IMO PSC Procedures and flag / class requirements.

How to use this checklist

Work top-to-bottom before arrival. Sections 0–2 cover what a PSCO reviews and observes on the initial inspection — clearing these prevents the “clear grounds” that escalate to a More Detailed Inspection (MDI). Sections 5.4–5.11 mirror the MDI itself, so you can rehearse exactly what an officer may be asked to demonstrate. Tick each item; treat any unticked box as an open action.

Tag legend D = detainable if found / failed (lack of maintenance is treated as non-accidental, §6.3). R = rectify before departure. MDI = a clear-grounds trigger that escalates to a detailed inspection.

0   Documents, logs & records	§3.1

| □ | Valid SOLAS Safety Construction Certificate (SAFCON), properly endorsed | §3.1.1 | R |
| --- | --- | --- | --- |
| □ | Oil Record Book — OWS use and sludge tank capacity entries consistent | §3.1.2 | R |
| □ | NOx Technical File (MARPOL VI Reg.13) matches engine & onboard spares | §3.1.2 | R |
| □ | Fuel change-over records (ECA transitions) and Bunker Delivery Notes complete | §3.1.2 | R |
| □ | SEEMP II/III Statement of Compliance — data submission & reporting year | §3.1.2 | R |
| □ | Latest Class Status Report — no overdue machinery-related items | §3.1.2 | R |
| □ | Conditions of Class / Conditions of Authority / exemptions identified | §3.1.3 | R |
| □ | UMS notation on Class cert; company UMS procedures available (if applicable) | §3.1.3.1 | R |
| □ | Documented maintenance system (PMS) in/under SMS, in crew working language | §3.1.3.2 | R |
| □ | Maintenance/repair/inspection records match Class report & NOx file dates | §3.1.3.3 | R |

1   Initial inspection & housekeeping — do not create clear grounds	§3.2

| □ | Engine room clean: no oil stains, oil-soaked lagging, oil in bilges / drip trays | §3.2.2.1 | R |
| --- | --- | --- | --- |
| □ | No temporary save-alls under leaking joints; oily rags removed after work | §3.2.2.1 | R |
| □ | Ventilation fully operational and effective | §3.2.2.2 | R |
| □ | Lighting operational and sufficient at all work locations | §3.2.2.3 | R |
| □ | Electrical equipment free of earth faults; insulation tests normal | §3.2.2.4 | R |
| □ | Piping, control / quick-closing / relief valves in good condition | §3.2.2.5 | R |
| □ | Control panel & monitoring free of abnormal or unjustifiably suppressed alarms | §3.2.2.6 | R |
| □ | Machinery free of cracks, corrosion, leaks, wear; isolation valves not hiding leaks | §3.2.2.7 | R |
| □ | Machinery-space escape routes clear of stores / waste | §3.2.2.8 | R |
| □ | Gauges undamaged, readings normal (start-air receiver adequate pressure) | §3.2.2.9 | R |
| □ | Boiler (UMS): two water-level indicators; safety systems operational | §3.2.2.10 | R |
| □ | Rubber expansion joints (REJs) in satisfactory condition | §3.2.2.11 | R |
| □ | Maintenance records accurate & verifiable; crew familiar and complying | §3.2.3 | R |

2   Clear-grounds triggers — know what escalates to a Detailed Inspection	§4.2

| □ | Notified propulsion failure or lack of available main-engine power | §4.2.1.1 | MDI |
| --- | --- | --- | --- |
| □ | Too few auxiliary generators available / inadequate load sharing | §4.2.1.2 | MDI |
| □ | Excessive oil leakage, oil-soaked lagging, oil in bilges or save-alls | §4.2.1.3 | MDI |
| □ | Crew unfamiliar with equipment testing per maker's instructions | §4.2.1.4 | MDI |
| □ | Excessive inhibited / off-scan / reposed / manually suppressed alarms | §4.2.1.5 | MDI |
| □ | Start-air receiver low, or too few air compressors in service | §4.2.1.6 | MDI |
| □ | Large earth fault on 220 V (≤ 0.5 MΩ) or any earth fault on 440 V | §4.2.1.7 | MDI |
| □ | Compartment cluttered with rubbish, oily rags, unsecured spare parts | §4.2.1.8 | MDI |
| □ | Inoperable automated control, alarm or safety systems | §4.2.1.9 | MDI |
| □ | Evidence of a recent emergency repair on critical machinery | §4.2.1.10 | MDI |

5.4   MDI — Main engine alarm systems	§5.4

| □ | Visual: panels, alarms, UPS, sensors undamaged; review historical / false alarms | §5.4.2.1 | R |
| --- | --- | --- | --- |
| □ | Lamp & sound test; alarms audible at bridge & ER; not unjustifiably inhibited | §5.4.2.2 | R |
| □ | Cyl. cooling-water HT and exhaust-gas HT alarms function (with crew / Class) | §5.4.3.1 | R |
| □ | Alarm-system power-supply failure alarm functions | §5.4.3.2 | R |
| □ | Low starting-air pressure alarm (bridge-controlled M/E) | §5.4.3.3 | R |
| □ | HP fuel jacketed-pipe leak alarm (keel laid on/after 1 Feb 1992) | §5.4.3.4 | R |
| □ | Crankcase oil-mist alarm / bearing-temp monitor (≥2250 kW or >300 mm bore, UMS) | §5.4.3.5 | R |
| □ | Auxiliary-pump auto-changeover alarm (LO / FO / CW pumps, UMS) | §5.4.3.6 | R |
| □ | Inoperable M/E alarm or safety system | §6.3.1.3 | D |

5.5   MDI — Main engine component maintenance	§5.5

| □ | Drawings, specs & technical data available and in the PMS schedule | §5.5.2.1 | R |
| --- | --- | --- | --- |
| □ | Visual: cleanliness, secured oil/water piping; no corrosion / cracks / leaks | §5.5.2.2 | R |
| □ | Simulated low-LO-pressure slowdown/shutdown & bearing-temp trip test | §5.5.2.3 | R |
| □ | Spare-parts adequacy assessment for reliability-critical equipment | §5.5.3.1 | R |
| □ | Spares match NOx Technical File (2008 NOx Code engines) | §5.5.3.2 | R |
| □ | Planned Maintenance System (PMS) up to date | §5.5.3.3 | R |
| □ | Crew familiar with component maintenance procedures | §5.5.3.4 | R |
| □ | Non-metallic (rubber) expansion joints below waterline surveyed / replaced | §5.5.3.6 | R |
| □ | Cannot start / reverse / regulate / stop, or cracked liner or cylinder head | §6.3.1.1 | D |

5.6   MDI — Starting systems (air / electric)	§5.6

| □ | Visual: compressors / receivers clean, covers intact, no air leaks; batteries sound | §5.6.2.1 | R |
| --- | --- | --- | --- |
| □ | Start valves, distributor, relief/safety valves, dryers tested; turning-gear interlock | §5.6.2.2 | R |
| □ | Crew familiar with operation & maintenance procedures | §5.6.3.1 | R |
| □ | Emergency start (emergency compressor & receiver) effective | §5.6.3.2 | R |
| □ | Gauges correct; safety valves / fusible plugs not leaking or failed | §5.6.3.3 | R |
| □ | Non-return / anti-backfire valves not damaged or failed | §5.6.3.4 | R |
| □ | Overpressure safety valves carry 5-year test tags, no paint / corrosion | §5.6.3.5 | R |
| □ | Start-air lines fully separated from compressor discharge lines | §5.6.3.6 | R |
| □ | Receiver drain valve free; discharges oil-water mix without blockage | §5.6.3.7 | R |
| □ | Compressed-air fault causing abnormal operation of other systems | §6.3.2 | D |

5.7   MDI — Power generation	§5.7

| □ | Logbook history; gen plan implemented; gens cycled, share load, even exhaust temps | §5.7.2.1 | R |
| --- | --- | --- | --- |
| □ | Visual: structure, piping, valves, instruments; exhaust smoke leaks; TC/governor oil | §5.7.2.2 | R |
| □ | FO/LO/CW/start-air tested; LO filter pressures; oil analysis; paralleling & load-shed | §5.7.2.3 | R |
| □ | FO isolation valves operate (keel on/after 1 Jul 2002) | §5.7.2.3 | R |
| □ | Provision to clean / inspect / maintain the generator | §5.7.3.1 | R |
| □ | Low-LO-pressure safety devices function | §5.7.3.2 | R |
| □ | One gen out of service: remaining sets supply propulsion / safety / habitability | §5.7.3.3 | R |
| □ | Remote & local control, sync & load-shedding; engineer familiar | §5.7.3.4 | R |
| □ | Main-generator failure with no maintain / immediate restore of power | §6.3.6.2 | D |

5.8   MDI — Electrical systems	§5.8

| □ | Switchboard clear; indicator lights & insulation alarms work (not hidden by tripped breaker) | §5.8.2.1 | R |
| --- | --- | --- | --- |
| □ | Insulated mats / back panels / handrails; equipment earthed; Ex protection in hazardous spaces | §5.8.2.1 | R |
| □ | Main source supplies all three phases (SOLAS II-1 Reg.41) | §5.8.2.2 | R |
| □ | Emergency gen auto-starts and on-load within 45 s; battery instantaneous (Reg.43.3) | §5.8.2.2 | R |
| □ | Emergency-gen fuel ≥ 18 h (cargo) / 36 h (passenger); gauges & records | §5.8.3.1 | R |
| □ | Auto-start energy for 3 consecutive starts + 3 more within 30 min (Reg.44.2) | §5.8.3.2 | R |
| □ | Overload protective device rating / setting permanently indicated at device | §5.8.3.3 | R |
| □ | Lighting fixtures secure; anti-vibration / waterproof where required | §5.8.3.4 | R |
| □ | On load: start emergency fire pump & steering pump to load-test emergency genset | §5.8.3.6 | R |
| □ | Failure of emergency power source, or loss/no-restore of main generator power | §6.3.6.2 | D |
| □ | Jumper cables used to isolate sensors / indicators on critical alarms | §6.3.6.2.5 | D |

5.9   MDI — Steering gear	§5.9

| □ | Records: calibration / testing; emergency steering drills at least every 3 months | §5.9.2.1 | R |
| --- | --- | --- | --- |
| □ | Visual: no leaks; reserve oil & level gauge; air vents & pressure gauges; linkages tight | §5.9.2.2 | R |
| □ | Self-closing level-gauge device not held open; no rudder-stock water ingress | §5.9.2.2 | R |
| □ | Prompt / accurate response, max rudder angle & slew rate; alarms & interlocks | §5.9.2.3 | R |
| □ | Power-cut triggers audible/visual alarm; auto-restart on power restoration | §5.9.3.1 | R |
| □ | No excess play between rudder stock and steering gear | §5.9.3.2 | R |
| □ | Local steering test; response time meets convention; limit switches function | §5.9.3.3 | R |
| □ | Bridge↔steering-flat comms; bridge angle indicator matches local reading | §5.9.3.5 | R |
| □ | Steering alarms (power / short-circuit / overload / phase) function | §5.9.3.6 | R |
| □ | Emergency steering drill performed | §5.9.3.7 | R |
| □ | Main response >28 s / aux >60 s, rudder angle unattainable, or alarm failure | §6.3.4.1 | D |

5.10   MDI — Boilers	§5.10

| □ | Records: survey cert, safety-valve calibration, water dosing / quality tests, chemicals | §5.10.2.1 | R |
| --- | --- | --- | --- |
| □ | Visual: no corrosion/cracks/leaks/smoke on body, burner, exhaust, lines, valves, manholes | §5.10.2.2 | R |
| □ | Burner operable; clean sight glass; no black smoke in exhaust | §5.10.2.2 | R |
| □ | Reliable under load; safety devices / alarms / interlocks / auto-control function | §5.10.2.3 | R |
| □ | Insulation intact; no pipeline leaks | §5.10.3.1 | R |
| □ | Main steam / sampling / drain / stop valves operate; no corrosion or failure | §5.10.3.2 | R |
| □ | Safety-valve easing gear correctly set, not cracked / corroded; safe outlet | §5.10.3.3 | R |
| □ | Unmanned oil-fired boiler shuts fuel + alarm on low water / air / flame failure | §5.10.3.4 | R |
| □ | Two water-level indications, ≥1 direct gauge glass working; remote SV operation | §5.10.3.5 | R |
| □ | Cracks / leaks rendering boiler inoperable (repair + RO survey before sailing) | §6.3.3 | D |

5.11   MDI — Unmanned machinery spaces (UMS)	§5.11

| □ | Minimum Safe Manning vs watch arrangement consistent with UMS notation | §5.11.2.1 | R |
| --- | --- | --- | --- |
| □ | Bridge: M/E control position, RPM & start-air gauges; sensor probes sound; clear LCDs | §5.11.2.2 | R |
| □ | All UMS functions reliable; alarms at ECR / wheelhouse / accommodation | §5.11.2.3 | R |
| □ | Main + emergency supply with reliable changeover; personnel / dead-man alarm (A.1021(26)) | §5.11.2.3 | R |
| □ | Bilge high-level alarm; alarm for prolonged / frequent bilge-pump operation | §5.11.3.1 | R |
| □ | Sea-inlet / below-waterline / bilge-injection valve controls reachable (remote if needed) | §5.11.3.2 | R |
| □ | CCR & extended alarm display/function; power-failure alarm; no locked-out alarms | §5.11.3.3 | R |
| □ | Engineers' alarm reset; personnel alarm auto-triggers if not reset ≤30 min; 3-min pre-warn | §5.11.3.4 | R |
| □ | Dual-pump auto switch; standby auto-starts + alarm when a pump stops | §5.11.3.5 | R |
| □ | Standby genset auto-start / sync / load-share / de-load / step-shed normal | §5.11.3.6 | R |
| □ | Auto-control loss preventing UMS operation with insufficient manning | §6.3.1.3 | D |

Reporting & detention logic (§6.2) A voluntarily reported malfunction with no resulting casualty should normally be rectified before departure rather than detained. Detention may still follow where onboard repairs are inadequate, a previously reported defect is not properly rectified, the same defect recurs, or the defect is a verifiable direct result of lack of maintenance (therefore not accidental). For critical deficiencies, detention should be considered.

Readiness self-score Open actions (unticked): ______ Detainable (D) items open: ______ Verified by / rank: ____________ Date: __________